(20/06/2026) till now we have the ph and ec sensor we  written a fsm based firmware for this project  the   process is like the ph and ec are measured at the start of switch condition and then if there is ec deficit it goes for nutrition dosing and them verify the does the dosing is sufficient if not it again starts the dosing until the dosing is perfectly matched the requirement and check for ph if it is deficit it will adujust the ph until it reaches the target value

(22/06/2026) today I have researched about the crop specific  ec and ph values for each crop while researching I have gone through a us based hydroponics hobby kit seller where I found the charts for  crop specific nutrition values and some other related information about hydroponics nutritions .

These is the information I have found in that page:

Measurement Definitions

There are 5 “measurement terms” that you’ll likely encounter when researching how to grow your plants     pH, TDS, EC, cF, and PPM. Here’s what each one means:

- pH (Potential for Hydrogen): Measure of how acidic or basic water is using a 14 point scale.

- TDS (Totally Dissolved Solids): Refers to the amount of substances — such as salts, minerals, metals, etc — that have been dissolved in water.

- EC (Electrical Conductivity): Measures how well a liquid can carry a current through it.

- PPM (Parts Per Million): Measures the concentration of elements in your nutrient solution.

- cF (Conductivity Factor): Measures electrically charged nutrient ions in your nutrient solution.

## Hydroponic Vegetable Chart: pH, EC, cF, and PPM

| Vegetable | pH | EC (mS/cm) | cF | PPM (700 Scale) | PPM (500 Scale) |
| --- | --- | --- | --- | --- | --- |
| Artichoke | 6.5 – 7.5 | 0.8 – 1.8 | 8 – 18 | 560 – 1260 | 400 – 900 |
| Asparagus | 6.0 – 6.8 | 1.4 – 1.8 | 14 – 18 | 980 – 1260 | 700 – 900 |
| Bean (Common) | 6.0 | 2.0 – 4.0 | 20 – 40 | 1400 – 2800 | 1000 – 2000 |
| Beans (Italian Bush) | 6.0 – 6.5 | — | — | — | — |
| Beans (Lima) | 6.0 – 6.5 | — | — | — | — |
| Beans (Pole) | 6.0 – 6.5 | — | — | — | — |
| Broad Bean | 6.0 – 6.5 | 1.8 – 2.2 | 18 – 22 | 1260 – 1540 | 900 – 1100 |
| Broccoli | 6.0 – 6.5 | 2.8 – 3.5 | 28 – 35 | 1960 – 2450 | 1400 – 1750 |
| Brussels Sprout | 6.5 – 7.5 | 2.5 – 3.0 | 25 – 30 | 1750 – 2100 | 1250 – 1500 |
| Cabbage | 6.5 – 7.0 | 2.5 – 3.0 | 25 – 30 | 1750 – 2100 | 1250 – 1500 |
| Capsicum | 6.0 – 6.5 | 1.8 – 2.2 | 18 – 22 | 1260 – 1540 | 900 – 1100 |
| Cauliflower | 6.0 – 7.0 | 0.5 – 2.0 | 5 – 20 | 350 – 1400 | 250 – 1000 |
| Celery | 6.5 | 1.8 – 2.4 | 18 – 24 | 1260 – 1680 | 900 – 1200 |
| Collard Greens | 6.5 – 7.5 | — | — | — | — |
| Cucumber | 5.8 – 6.0 | 1.7 – 2.5 | 17 – 25 | 1190 – 1750 | 850 – 1250 |
| Eggplant | 5.5 | 2.5 – 3.5 | 25 – 35 | 1750 – 2450 | 1250 – 1750 |
| Endive | 6.0 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Fodder | 6.0 | 1.8 – 2.0 | 18 – 20 | 1260 – 1400 | 900 – 1000 |
| Lettuce | 5.5 – 6.5 | 0.8 – 1.2 | 8 – 12 | 560 – 840 | 400 – 600 |
| Marrow | 6.0 | 1.8 – 2.4 | 18 – 24 | 1260 – 1680 | 900 – 1200 |
| Okra | 6.5 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Pak-choi | 7.0 | 1.5 – 2.0 | 15 – 20 | 1050 – 1400 | 750 – 1000 |
| Pea | 6.0 – 7.0 | 0.8 – 1.8 | 8 – 18 | 560 – 1260 | 400 – 900 |
| Peas (Sugar) | 6.0 – 6.8 | — | — | — | — |
| Pepino | 6.0 – 6.5 | 2.0 – 5.0 | 20 – 50 | 1400 – 3500 | 1000 – 2500 |
| Peppers | 5.8 – 6.3 | 2.0 – 3.0 | 20 – 30 | 1400 – 2100 | 1000 – 1500 |
| Bell Peppers | 6.0 – 6.5 | 2.0 – 2.5 | 20 – 25 | 1400 – 1750 | 1000 – 1250 |
| Hot Peppers | 6.0 – 6.5 | 3.0 – 3.5 | 30 – 35 | 2100 – 2450 | 1500 – 1750 |
| Pumpkin | 5.5 – 7.5 | 1.8 – 2.4 | 18 – 24 | 1260 – 1680 | 900 – 1200 |
| Radicchio | 5.5 – 6.5 | — | — | — | — |
| Spinach | 5.5 – 6.6 | 1.8 – 2.3 | 18 – 23 | 1260 – 1610 | 900 – 1150 |
| Sweet Corn | 6.0 | 1.6 – 2.4 | 16 – 24 | 1120 – 1680 | 800 – 1200 |
| Swiss Chard | 6.0 – 6.5 | 1.6 – 2.3 | 16 – 23 | 1120 – 1610 | 800 – 1150 |
| Squash (Summer) | 5.0 – 6.5 | 1.8 – 2.4 | 18 – 24 | 1260 – 1680 | 900 – 1200 |
| Squash (Winter) | 5.0 – 6.5 | — | — | — | — |
| Taro | 5.0 – 5.5 | 2.5 – 3.0 | 25 – 30 | 1750 – 2100 | 1250 – 1500 |
| Tomato | 5.5 – 6.5 | 2.0 – 5.0 | 20 – 50 | 1400 – 3500 | 1000 – 2500 |
| Zucchini | 6.0 | 1.8 – 2.4 | 18 – 24 | 1260 – 1680 | 900 – 1200 |

Hydroponic Root Vegetable Chart: pH, EC, cF, and PPM

| Herb | pH | EC (mS/cm) | cF | PPM (700 Scale) | PPM (500 Scale) |
| --- | --- | --- | --- | --- | --- |
| Basil | 5.5 – 6.5 | 1.0 – 1.6 | 10 – 16 | 700 – 1120 | 500 – 800 |
| Chicory | 5.5 – 6.0 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Chives | 6.0 – 6.5 | 1.8 – 2.2 | 18 – 22 | 1260 – 1540 | 900 – 1100 |
| Fennel | 6.4 – 6.8 | 1.0 – 1.4 | 10 – 14 | 700 – 980 | 500 – 700 |
| Lavender | 6.4 – 6.8 | 1.0 – 1.4 | 10 – 14 | 700 – 980 | 500 – 700 |
| Lemon Balm | 5.5 – 6.5 | 1.0 – 1.6 | 10 – 16 | 700 – 1120 | 500 – 800 |
| Marjoram | 6.0 | 1.6 – 2.0 | 16 – 20 | 1120 – 1400 | 800 – 1000 |
| Mint | 5.5 – 6.0 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Mustard Cress | 6.0 – 6.5 | 1.2 – 2.4 | 12 – 24 | 840 – 1680 | 600 – 1200 |
| Parsley | 5.5 – 6.0 | 0.8 – 1.8 | 8 – 18 | 560 – 1260 | 400 – 900 |
| Rosemary | 5.5 – 6.0 | 1.0 – 1.6 | 10 – 16 | 700 – 1120 | 500 – 800 |
| Sage | 5.5 – 6.5 | 1.0 – 1.6 | 10 – 16 | 700 – 1120 | 500 – 800 |
| Thyme | 5.5 – 7.0 | 0.8 – 1.6 | 8 – 16 | 560 – 1120 | 400 – 800 |
| Watercress | 6.5 – 6.8 | 0.4 – 1.8 | 4 – 18 | 280 – 1260 | 200 – 900 |

Hydroponic Fruits Chart: pH, EC, cF, and PPM

| Fruit | pH | EC (mS/cm) | cF | PPM (700 Scale) | PPM (500 Scale) |
| --- | --- | --- | --- | --- | --- |
| Banana | 5.5 – 6.5 | 1.8 – 2.2 | 18 – 22 | 1260 – 1540 | 900 – 1100 |
| Black Currant | 6.0 | 1.4 – 1.8 | 14 – 18 | 980 – 1260 | 700 – 900 |
| Blueberry | 4.0 – 5.0 | 1.8 – 2.0 | 18 – 20 | 1260 – 1400 | 900 – 1000 |
| Melon | 5.5 – 6.0 | 2.0 – 2.5 | 20 – 25 | 1400 – 1750 | 1000 – 1250 |
| Passionfruit | 6.5 | 1.6 – 2.4 | 16 – 24 | 1120 – 1680 | 800 – 1200 |
| Paw-Paw (Papaya) | 6.5 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Pineapple | 5.5 – 6.0 | 2.0 – 2.4 | 20 – 24 | 1400 – 1680 | 1000 – 1200 |
| Red Currant | 6.0 | 1.4 – 1.8 | 14 – 18 | 980 – 1260 | 700 – 900 |
| Rhubarb | 5.0 – 6.0 | 1.6 – 2.0 | 16 – 20 | 1120 – 1400 | 800 – 1000 |
| Strawberries | 5.5 – 6.5 | 1.8 – 2.2 | 18 – 22 | 1260 – 1540 | 900 – 1100 |
| Watermelon | 5.8 | 1.5 – 2.4 | 15 – 24 | 1050 – 1680 | 750 – 1200 |

So  here we only got the ppm values we doesn’t know  what nutrition at what quantity . which nutrition we have to use at what quantity depends on the crop, growth stage ,environmental conditions.

Note:  There is also an important thing the water we get from borewell already have a ppm around 200-300 so they already contain some nutritions or minerals  so we have to first know the chemistry of the water so that we can adjust the nutrition composition accordingly

*The next task is to to find the cropspecific nutrition composition data *

(23/06/2026)

So upon reaserching I have found that these crops are devided into five different types based on there nutrition recipe the types are given below :

- Leafy Greens, Herbs & Foliage Crops

- Heavy-Feeding Fruiting Crops

- Balanced Vining, Melons & Pod Crops

- Root & Tuber Vegetables

- Perennial Berries (Salt-Sensitive)

These are the nutrition recipe based catogories of crops and for micronutrition there is a universal recepie that is given below:

Universal Micronutrient Targets (All Crop Categories)

| Micronutrient | Target PPM |
| --- | --- |
| Iron (Fe) | 2.0 |
| Manganese (Mn) | 0.5 |
| Boron (B) | 0.4 |
| Zinc (Zn) | 0.15 |
| Copper (Cu) | 0.03 |
| Molybdenum (Mo) | 0.02 |

- Leafy Greens, Herbs & Foliage Crops

| Element (PPM) | 🌱 Stage 1: Seedling / Early Root | 🌿 Stage 2: Full Vegetative / Maturity |
| --- | --- | --- |
| Nitrogen (N) | 50–70 | 150–165 |
| Phosphorus (P) | 15–20 | 30–40 |
| Potassium (K) | 60–80 | 200–210 |
| Calcium (Ca) | 40–50 | 90–100 |
| Magnesium (Mg) | 12–15 | 24–35 |
| Sulfur (S) | 15–20 | 30–45 |
| Target EC | 0.5–0.8 mS/cm | 1.2–1.6 mS/cm |
| Target PPM (500 Scale) | 250–400 | 600–800 |

Original Reference

Source: Cornell University Controlled Environment Agriculture (CEA) Hydroponic Lettuce Handbook

Location in Document: Chapter 1 – System Components, under the section "Pond Solution" (Pages 10–13).

Raw Nutrient Targets Reported by Cornell

Cornell specifies nutrient concentrations in millimoles per liter (mmol/L).

| Nutrient | Target Concentration |
| --- | --- |
| Nitrate-Nitrogen (NO₃⁻-N) | 11.0 mmol/L |
| Phosphorus (P) | 1.0 mmol/L |
| Potassium (K) | 5.4 mmol/L |
| Calcium (Ca) | 2.25 mmol/L |
| Magnesium (Mg) | 1.0 mmol/L |

Conversion Method

To convert nutrient concentrations from mmol/L to absolute elemental PPM (mg/L), the following equation is used:

PPM (mg/L) = mmol/L × Atomic Weight

Calculations

Nitrogen (N) 11.0 mmol/L × 14.007 g/mol = 154.0 ppm

Phosphorus (P) 1.0 mmol/L × 30.974 g/mol = 31.0 ppm

Potassium (K) 5.4 mmol/L × 39.098 g/mol = 211.1 ppm

Calcium (Ca) 2.25 mmol/L × 40.078 g/mol = 90.1 ppm

Magnesium (Mg) 1.0 mmol/L × 24.305 g/mol = 24.3 ppm

Resulting Elemental Nutrient Profile

| Element | Calculated Target (ppm) |
| --- | --- |
| Nitrogen (N) | 154.0 |
| Phosphorus (P) | 31.0 |
| Potassium (K) | 211.1 |
| Calcium (Ca) | 90.1 |
| Magnesium (Mg) | 24.3 |

For practical hydroponic implementation, the Cornell nutrient profile can be represented as:

| Element | Target Range (ppm) |
| --- | --- |
| Nitrogen (N) | 150–165 |
| Phosphorus (P) | 31 |
| Potassium (K) | 210 |
| Calcium (Ca) | 90 |
| Magnesium (Mg) | 24 |

These values form the baseline nutrient solution commonly used for leafy greens such as lettuce, spinach, basil, Swiss chard, pak choi, and other vegetative crops in controlled environment agriculture systems.

- Heavy-Feeding Fruiting Crops

| Element (PPM) | 🌱 Stage 1: Seedling | 🌿 Stage 2: Vegetative Growth | 🌼 Stage 3: Flowering & Fruit Set | 🍅 Stage 4: Heavy Fruit Ripening |
| --- | --- | --- | --- | --- |
| Nitrogen (N) | 50 | 150 | 140 | 130 |
| Phosphorus (P) | 20 | 40 | 50 | 50 |
| Potassium (K) | 70 | 200 | 280 | 350 |
| Calcium (Ca) | 50 | 130 | 170 | 200 |
| Magnesium (Mg) | 15 | 45 | 60 | 70 |
| Sulfur (S) | 20 | 50 | 80 | 95 |
| Target EC | 0.6 | 1.6 | 2.2 | 2.8–3.2 |
| Target PPM (500 Scale) | 300 | 800 | 1100 | 1400–1600 |

Original Reference

Source: University of Arizona Controlled Environment Agriculture Center (CEAC)

Document Section: Chapter 10 – Nutrient Solutions

Location: Pages 10-3 to 10-5, including Table 10.2

Nutrient Management Strategy

The University of Arizona CEAC recommends adjusting nutrient concentrations according to the crop's developmental stage rather than maintaining a single nutrient recipe throughout the entire growing cycle.

Tomato and eggplant plants undergo major physiological changes as they progress from vegetative growth to flowering and fruit production. As a result, nutrient requirements and target EC values are modified throughout the crop cycle.

Stage 1 – Vegetative Growth (Weeks 0–6)

Objective

Develop strong roots, stems, and foliage before flowering begins.

Recommended Conditions

| Parameter | Value |
| --- | --- |
| Target EC | 1.5 mS/cm |
| Nitrogen (N) | Moderate |
| Phosphorus (P) | Moderate |
| Potassium (K) | Moderate |
| Calcium (Ca) | Moderate |
| Magnesium (Mg) | Moderate |

At this stage, the focus is on building plant structure and leaf area rather than fruit production.

Stage 2 – Generative / Fruit-Set Stage (Weeks 6–12)

Objective

Support flowering, pollination, and early fruit development.

Recommended Conditions

| Parameter | Value |
| --- | --- |
| Target EC | 2.0 mS/cm |
| Potassium Requirement | Increased |
| Calcium Requirement | Increased |
| Nitrogen Requirement | Maintained |

The nutrient solution is strengthened to support flower formation and initial fruit growth.

Stage 3 – Heavy Fruit Ripening (Weeks 12+)

Objective

Maximize fruit size, sugar accumulation, firmness, and overall yield.

Recommended Elemental Targets

| Element | Target Concentration (PPM) |
| --- | --- |
| Nitrogen (N) | 140–150 |
| Phosphorus (P) | 50 |
| Potassium (K) | 300–350 |
| Calcium (Ca) | 170–200 |
| Magnesium (Mg) | 60–70 |

Target EC

| Parameter | Value |
| --- | --- |
| EC | 2.8–3.2 mS/cm |
| Equivalent PPM (500 Scale) | 1400–1600 |

Scientific Basis

During fruit ripening, tomato plants absorb significantly larger amounts of potassium.

Potassium plays a critical role in:

- Sugar transport within the plant

- Fruit expansion

- Color development

- Fruit firmness

- Overall yield quality

As fruit clusters enlarge, potassium requirements increase from approximately 200 ppm during vegetative growth to as much as 350 ppm during heavy fruit production.

Insufficient potassium during this stage may result in:

- Reduced fruit size

- Lower sugar content

- Uneven ripening

- Internal fruit hollows

- Reduced market quality

Calcium levels are also increased to help prevent physiological disorders such as blossom-end rot and to improve fruit firmness.

Stage 1 – Seedling

| Element | PPM |
| --- | --- |
| N | 50 |
| P | 20 |
| K | 70 |
| Ca | 50 |
| Mg | 15 |

Stage 2 – Vegetative Growth

| Element | PPM |
| --- | --- |
| N | 150 |
| P | 40 |
| K | 200 |
| Ca | 130 |
| Mg | 45 |

Stage 3 – Flowering & Fruit Set

| Element | PPM |
| --- | --- |
| N | 140 |
| P | 50 |
| K | 280 |
| Ca | 170 |
| Mg | 60 |

Stage 4 – Heavy Fruit Ripening

| Element | PPM |
| --- | --- |
| N | 130–150 |
| P | 50 |
| K | 300–350 |
| Ca | 170–200 |
| Mg | 60–70 |

These values can be used as the nutrient profile for tomato, eggplant, and similar heavy-feeding fruiting crops within the  nutrient recommendation engine.

- Balanced Vining, Melons & Pod Crops

| Element (PPM) | 🌱 Stage 1: Seedling / Early Root | 🌿 Stage 2: Vegetative Vine Growth | 🫑 Stage 3: Flowering & Continuous Harvest |
| --- | --- | --- | --- |
| Nitrogen (N) | 60 | 160 | 185 |
| Phosphorus (P) | 20 | 40 | 45 |
| Potassium (K) | 70 | 200 | 260 |
| Calcium (Ca) | 50 | 120 | 160 |
| Magnesium (Mg) | 15 | 40 | 55 |
| Sulfur (S) | 20 | 50 | 70 |
| Target EC | 0.6 | 1.6 | 2.2–2.5 |
| Target PPM (500 Scale) | 300 | 800 | 1100–1250 |

Original Reference

Source: Greenhouse Crop Nutrient Management Guidelines (C. Cook)

Location in Document: Page 1, Table 1 and Table 2 – Pepper/Capsicum Formulation Layout

Nutrient Management Strategy

Unlike tomatoes, vining and continuous-harvest crops must simultaneously support:

- Ongoing vegetative growth

- Continuous flowering

- Fruit development

- New vine extension

Because these crops continue producing leaves and stems while carrying fruit, nitrogen levels must remain relatively high throughout the production cycle.

Elemental Nutrient Profile

The greenhouse nutrient management guide provides the following elemental nutrient targets for pepper-family crops:

| Element | Target Concentration (PPM) |
| --- | --- |
| Nitrate Nitrogen (NO₃-N) | 190 |
| Ammonium Nitrogen (NH₄-N) | 18 |
| Total Nitrogen (N) | 208 |
| Phosphorus (P) | 40 |
| Potassium (K) | 340 |
| Calcium (Ca) | 170 |
| Magnesium (Mg) | 50 |
| Sulfate (SO₄) | 360 |

Sulfur Conversion

The reference reports sulfur in the form of sulfate (SO₄).

To determine elemental sulfur:

Elemental Sulfur = Sulfate × (32.06 ÷ 96.06)

Calculation:

360 × (32.06 ÷ 96.06) = 120 ppm elemental sulfur

For practical hydroponic nutrient formulation, sulfur is generally maintained within the range of approximately 70–120 ppm depending on crop stage and environmental conditions.

Scientific Basis

Vining crops continue expanding their vegetative structure throughout much of their productive life.

Examples include:

- Bell Peppers

- Hot Peppers

- Cucumbers

- Melons

- Watermelons

- Squash

- Pumpkin

- Beans

- Peas

Because these crops maintain active vegetative growth while simultaneously producing fruit:

- Nitrogen demand remains elevated.

- Potassium demand increases significantly during flowering and fruit development.

- Calcium remains critical for fruit quality and structural integrity.

- Magnesium supports chlorophyll production and photosynthesis.

Reducing nitrogen too aggressively during fruiting can lead to:

- Reduced vine growth

- Smaller canopy size

- Lower flower production

- Reduced harvest duration

- Lower overall yield

Stage 1 – Seedling / Early Root Development

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 60 |
| Phosphorus (P) | 20 |
| Potassium (K) | 70 |
| Calcium (Ca) | 50 |
| Magnesium (Mg) | 15 |
| Sulfur (S) | 20 |

Target EC: 0.6 mS/cm

Target PPM (500 Scale): 300

Stage 2 – Vegetative Vine Growth

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 160 |
| Phosphorus (P) | 40 |
| Potassium (K) | 200 |
| Calcium (Ca) | 120 |
| Magnesium (Mg) | 40 |
| Sulfur (S) | 50 |

Target EC: 1.6 mS/cm

Target PPM (500 Scale): 800

Stage 3 – Flowering & Continuous Harvest

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 185 |
| Phosphorus (P) | 45 |
| Potassium (K) | 260 |
| Calcium (Ca) | 160 |
| Magnesium (Mg) | 55 |
| Sulfur (S) | 70 |

Target EC: 2.2–2.5 mS/cm

Target PPM (500 Scale): 1100–1250

Crop Category Coverage

This nutrient profile can be applied to:

- Bell Peppers

- Hot Peppers

- Cucumbers

- Zucchini

- Marrow

- Summer Squash

- Winter Squash

- Pumpkin

- Melons

- Watermelons

- Beans

- Peas

- Sweet Corn

The profile serves as the baseline recommendation for Balanced Vining Crop nutrient category and can be adjusted automatically based on crop stage, water chemistry, environmental conditions, and reservoir size.

- Root & Tuber Vegetables

| Element (PPM) | 🌱 Stage 1: Canopy Establishment | 🥕 Stage 2: Root Initiation & Bulking |
| --- | --- | --- |
| Nitrogen (N) | 130 | 75 |
| Phosphorus (P) | 40 | 65 |
| Potassium (K) | 180 | 280 |
| Calcium (Ca) | 100 | 110 |
| Magnesium (Mg) | 35 | 45 |
| Sulfur (S) | 40 | 55 |
| Target EC | 1.3 | 1.8–2.2 |
| Target PPM (500 Scale) | 650 | 900–1100 |

Original Reference

Source: Nutrient Solutions for Greenhouse Crops
Authors: C. Sonneveld and Geerten van der Lugt

Location in Document: Section B – Crop-Specific Standard Directives, Chapter 5 (Pages 28–34)

Nutrient Management Strategy

This reference contains crop-specific nutrient solution recommendations developed from greenhouse experiments conducted by C. Sonneveld and colleagues.

The nutrient profiles are designed for crops where the harvested product is a storage organ (root, tuber, bulb, or rhizome) rather than leaves or fruit.

Examples include:

- Beetroot

- Carrot

- Radish

- Turnip

- Parsnip

- Onion

- Garlic

- Potato

- Sweet Potato

The same principles are also applied, with modifications, to certain perennial and tropical crops.

Key Findings from the Research

Nitrogen Management

The research indicates that excessive nitrogen promotes vegetative growth at the expense of root development.

When nitrogen concentrations exceed:

6.0 mmol/L Nitrogen

Conversion:

6.0 mmol/L × 14.007 = 84 ppm Nitrogen

Potential effects include:

- Excessive leaf growth

- Reduced starch accumulation

- Smaller storage roots

- Reduced root quality

- Lower harvest yields

Phosphorus Management

The research recommends elevated phosphorus concentrations during root initiation and bulking phases.

Target phosphorus level:

1.5 mmol/L Phosphorus

Conversion:

1.5 mmol/L × 30.974 = 46.5 ppm Phosphorus

Operational greenhouse recommendations typically maintain:

46–65 ppm Phosphorus

Benefits include:

- Increased root cell division

- Improved root expansion

- Enhanced energy transfer within the plant

- Better root initiation

Potassium Management

Potassium is maintained at relatively high concentrations throughout root development.

Target potassium level:

7.5 mmol/L Potassium

Conversion:

7.5 mmol/L × 39.098 = 293.2 ppm Potassium

Benefits include:

- Carbohydrate transport

- Starch accumulation

- Sugar movement into storage tissues

- Improved root size and weight

- Enhanced crop quality

Converted Elemental Targets

| Element | mmol/L | Converted PPM |
| --- | --- | --- |
| Nitrogen (N) | 6.0 | 84 |
| Phosphorus (P) | 1.5 | 46.5 |
| Potassium (K) | 7.5 | 293.2 |

Scientific Interpretation

Root crops differ significantly from leafy and fruiting crops.

The nutrient strategy shifts from:

"Build leaves"

to

"Store carbohydrates underground."

To achieve this:

- Nitrogen is reduced.

- Phosphorus is increased.

- Potassium becomes the dominant nutrient.

This encourages the plant to direct sugars and energy into root enlargement rather than producing excessive foliage.

Stage 1 – Canopy Establishment

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 130 |
| Phosphorus (P) | 40 |
| Potassium (K) | 180 |
| Calcium (Ca) | 100 |
| Magnesium (Mg) | 35 |
| Sulfur (S) | 40 |

Target EC: 1.3 mS/cm

Target PPM (500 Scale): 650

Stage 2 – Root Initiation & Bulking

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 75 |
| Phosphorus (P) | 65 |
| Potassium (K) | 280 |
| Calcium (Ca) | 110 |
| Magnesium (Mg) | 45 |
| Sulfur (S) | 55 |

Target EC: 1.8–2.2 mS/cm

Target PPM (500 Scale): 900–1100

Applicable Crop Group

- Beetroot

- Carrot

- Garlic

- Leek

- Onion

- Parsnip

- Potato

- Sweet Potato

- Radish

- Turnip

- Taro

These nutrient profiles provide the baseline recommendations used by ORIVA for automated nutrient dosing, growth-stage adjustments, and crop-specific nutrient management.

- Perennial Berries (Salt-Sensitive)

| Element (PPM) | 🌱 Stage 1: Early Leafing | 🍓 Stage 2: Flowering & Fruit Production |
| --- | --- | --- |
| Nitrogen (N) | 60 | 90 |
| Phosphorus (P) | 25 | 35 |
| Potassium (K) | 90 | 150 |
| Calcium (Ca) | 80 | 120 |
| Magnesium (Mg) | 25 | 35 |
| Sulfur (S) | 30 | 45 |
| Target EC | 0.7 | 1.1–1.4 |
| Target PPM (500 Scale) | 350 | 550–700 |

Original Reference

Source: Nutrient Solutions for Greenhouse Crops
Authors: C. Sonneveld and Geerten van der Lugt
Publisher: Yara International

Location in Document: Chapter 5, Page 32

Table Reference: Table 5.4 – Standard Nutrient Solutions for Strawberries in Substrates

Nutrient Management Strategy

Perennial berry crops possess highly sensitive root systems and generally require lower nutrient concentrations than most vegetables and fruiting crops.

Examples include:

- Strawberry

- Blueberry

- Black Currant

- Red Currant

Excessive nutrient concentrations can result in:

- Root burn

- Reduced fruit quality

- Salt stress

- Lower yields

For this reason, commercial berry production typically operates at lower Electrical Conductivity (EC) values than tomato, pepper, or cucumber production systems.

Standard Nutrient Solution

The reference specifies a low-salinity nutrient solution with a target:

Electrical Conductivity (EC): 1.25 mS/cm

The nutrient solution is defined using millimoles per liter (mmol/L).

| Nutrient Ion | Concentration (mmol/L) |
| --- | --- |
| Nitrate-Nitrogen (NO₃-N) | 6.5 |
| Phosphorus (H₂PO₄⁻) | 1.0 |
| Potassium (K⁺) | 3.5 |
| Calcium (Ca²⁺) | 2.25 |
| Magnesium (Mg²⁺) | 1.0 |
| Sulfate (SO₄²⁻) | 1.1 |

Conversion to Elemental PPM

The conversion formula is:

PPM (mg/L) = mmol/L × Atomic Weight

Nitrogen (N)

6.5 × 14.007 = 91.0 ppm

Phosphorus (P)

1.0 × 30.974 = 31.0 ppm

Potassium (K)

3.5 × 39.098 = 136.8 ppm

Calcium (Ca)

2.25 × 40.078 = 90.2 ppm

Magnesium (Mg)

1.0 × 24.305= 24.3 ppm

Sulfur (S)

  - × 32.06 = 35.3 ppm

Converted Elemental Nutrient Profile

| Element | Calculated Target (PPM) |
| --- | --- |
| Nitrogen (N) | 91.0 |
| Phosphorus (P) | 31.0 |
| Potassium (K) | 136.8 |
| Calcium (Ca) | 90.2 |
| Magnesium (Mg) | 24.3 |
| Sulfur (S) | 35.3 |

Scientific Interpretation

Berry crops require a nutrient balance that prioritizes:

- Moderate nitrogen for controlled vegetative growth

- Sufficient potassium for fruit quality and sweetness

- Adequate calcium for fruit firmness

- Lower overall salinity to protect sensitive roots

Compared with tomatoes or peppers, berry crops are managed with:

- Lower EC

- Lower potassium

- Lower nitrogen

- Reduced total dissolved salts

This allows healthy fruit production while minimizing root-zone stress.

Stage 1 – Early Leafing

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 60 |
| Phosphorus (P) | 25 |
| Potassium (K) | 90 |
| Calcium (Ca) | 80 |
| Magnesium (Mg) | 25 |
| Sulfur (S) | 30 |

Target EC: 0.7 mS/cm

Target PPM (500 Scale): 350

Stage 2 – Flowering & Fruit Production

| Element | Target PPM |
| --- | --- |
| Nitrogen (N) | 90 |
| Phosphorus (P) | 35 |
| Potassium (K) | 150 |
| Calcium (Ca) | 120 |
| Magnesium (Mg) | 35 |
| Sulfur (S) | 45 |

Target EC: 1.1–1.4 mS/cm

Target PPM (500 Scale): 550–700

Applicable Crop Group

- Strawberry

- Blueberry

- Black Currant

- Red Currant

These nutrient profiles provide the baseline recommendations used for automated nutrient dosing, growth-stage adjustments, and crop-specific nutrient management in salt-sensitive berry crops.

Water Sutability Analysis (24/06/2026)

Has we discussed earlier the water we usually get it is not a pure water we will have some impurities in it .

The impurities means here are the minerals which from the  soil when the water travels through it like magnesium chlorides calcium etc… so the water will have some ppm and whether we can use this water or not depends on the water chemistry what minerals does water contain and  at what quantity does the ppm is in the sutable range or not .

  - The first thing to check the water sutability is to check the ppm or ec of the water if the ec is too high then the water is not sutable to use directly we have to purifythe water first and then use them.

  - The next important point to check is the water may behaving sutable ec or exactly in the same range we wanted but contains minerals that are not usefull for or plants although they have the sutable ec we cannot add usefull nutrients because the ec is already in range and the nutrition causes increase in ec which causes nutrients lockout .

So for the nutrition dosing system to work properly we need to the input current water chemistry and analyse the sutability and analysing the mineral composion and verying whethere we can achieve the required nutrition composition .

Below is the algorithm for water sutability analysis(25/06/2026) :

WATER SUITABILITY ANALYSIS ALGORITHM

Objective:
Determine whether the available water source is suitable for the selected crop and growth stage.

- Input Collection
• Select Crop Type
• Select Growth Stage
• Upload Water Chemistry Report

- pH

- EC

- Ca

- Mg

- K

- NO₃

- SO₄

- Na

- Cl

- HCO₃

- Load Crop Requirements
Retrieve crop-specific nutrient targets, pH range, and EC range from the ORIVA database.

- Water Analysis
• Identify beneficial nutrients already present
• Identify harmful ions (Na, Cl, HCO₃)
• Check pH suitability

- Nutrient Deficit Calculation
For each nutrient:

Deficit = Target Value − Available Value

If Deficit < 0, set Deficit = 0.

- Recipe Generation(there is a algorithm for recepie generation also we will dicuss after this)
Calculate the required fertilizer quantities to compensate for nutrient deficits.

- Solution Preparation
• Dose calculated nutrients into the reservoir
• Mix thoroughly using the circulation pump

- Validation
Measure:
• EC
• pH

Compare measured values with crop targets.

- Suitability Decision
Suitable:
• EC within target range
• pH within target range
• Harmful ions within acceptable limits

Correction Required:
• EC or pH outside target range
• Excessive harmful ions detected

- Output
• Water Suitability Status
• Measured EC
• Measured pH
• Nutrient Recipe
• Warning Messages
• Recommended Actions

Workflow:

Water Report
↓
Load Crop Requirements
↓
Analyze Water
↓
Calculate Nutrient Deficits
↓
Generate Recipe
↓
Dose & Mix Nutrients
↓
Measure EC & pH
↓
Suitability Decision
↓
Recommendations

Before the water even entering the plants we first check the compatability then we fix a nutrition recepie for it next we will see how we are going to generate the recepie

Recipe Generation Algorithm Using Matrix Optimization

Objective

The objective of this algorithm is to calculate the exact amount of each fertilizer required to satisfy the crop's nutrient requirements while considering the nutrients already present in the source water.

Instead of calculating each fertilizer individually, the algorithm solves all fertilizer quantities simultaneously using matrix optimization. This minimizes nutrient errors caused by fertilizers supplying multiple nutrients.

Workflow

Step 1: Load Crop Nutrient Profile

Retrieve the nutrient requirements for the selected crop and growth stage from the ORIVA crop database.

Example:

| Nutrient | Target (ppm) |
| --- | --- |
| Nitrogen | 150 |
| Phosphorus | 40 |
| Potassium | 220 |
| Calcium | 120 |
| Magnesium | 45 |
| Sulfur | 60 |

Step 2: Read Water Chemistry Report

Extract the nutrient concentrations already present in the source water.

Example:

| Nutrient | Available (ppm) |
| --- | --- |
| Nitrogen | 15 |
| Phosphorus | 5 |
| Potassium | 25 |
| Calcium | 80 |
| Magnesium | 20 |
| Sulfur | 15 |

Step 3: Calculate Nutrient Deficits

Calculate the additional nutrients required.

Required Nutrient = Target Nutrient − Available Nutrient

Example:

| Nutrient | Required (ppm) |
| --- | --- |
| Nitrogen | 135 |
| Phosphorus | 35 |
| Potassium | 195 |
| Calcium | 40 |
| Magnesium | 25 |
| Sulfur | 45 |

If any value is negative, set it to zero.

Step 4: Convert ppm to Total Nutrient Mass

Since

1 ppm = 1 mg/L

Multiply each required ppm by the reservoir volume.

Example:

Reservoir Volume = 100 L

Nitrogen Required

135 × 100

= 13,500 mg

= 13.5 g

Repeat for every nutrient.

Step 5: Build the Fertilizer Composition Matrix

Each fertilizer contributes different nutrients.

| Nutrient | Calcium Nitrate | Potassium Nitrate | MKP | Magnesium Sulfate | Micronutrients | Potassium Sulfate |
| --- | --- | --- | --- | --- | --- | --- |
| Nitrogen | 0.155 | 0.130 | 0.000 | 0.000 | 0.000 | 0.000 |
| Phosphorus | 0.000 | 0.000 | 0.227 | 0.000 | 0.000 | 0.000 |
| Potassium | 0.000 | 0.387 | 0.287 | 0.000 | 0.000 | 0.448 |
| Calcium | 0.190 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Magnesium | 0.000 | 0.000 | 0.000 | 0.098 | 0.000 | 0.000 |
| Sulfur | 0.000 | 0.000 | 0.000 | 0.130 | 0.000 | 0.184 |

Each value represents the grams of nutrient supplied by one gram of fertilizer.

Step 6: Solve the Optimization Problem

The unknown fertilizer quantities are represented as:

- x₁ = Calcium Nitrate

- x₂ = Potassium Nitrate

- x₃ = MKP

- x₄ = Magnesium Sulfate

- x₅ = Micronutrient Mix

- x₆ = Potassium Sulfate

The optimization solves

A × X ≈ B

Where:

- A = Fertilizer composition matrix

- X = Unknown fertilizer quantities

- B = Required nutrient mass

Subject to:

- Fertilizer quantities ≥ 0

- Nutrient error is minimized

The optimization computes the best fertilizer combination that satisfies all nutrient requirements simultaneously.

Step 7: Generate the Fertilizer Recipe

The optimized solution returns the required fertilizer quantities.

Example:

| Fertilizer | Quantity |
| --- | --- |
| Calcium Nitrate | 92.4 g |
| Potassium Nitrate | 55.6 g |
| MKP | 21.8 g |
| Magnesium Sulfate | 48.3 g |
| Micronutrient Mix | 8.0 g |
| Potassium Sulfate | 17.5 g |

Step 8: Convert to Stock Solution Volumes

Each fertilizer is prepared as a concentrated stock solution.

The required pump volume is calculated using:

Pump Volume (mL) = Required Fertilizer (g) ÷ Stock Concentration (g/mL)

These pump volumes are then sent to the ESP32 dosing controller.

Final Output

The Recipe Generation Module outputs:

- Corrected nutrient requirements

- Fertilizer quantities (g)

- Stock solution dosing volumes (mL)

- Final nutrient recipe

- Data for the automatic dosing system